Ralink WPS (WiFi Portected Setup) UPNP Daemon HOWTO

- Build WPS UPnP daemon

Requirements:

(1). Linux platform

(2). Ralink wireless driver version which support WPS

(3). Libupnp

You can download the libupnp source code from the following URL: http://upnp.sourceforge.net/

libupnp-1.3.1 is preferred version. For other versions, you may need to patch our modification to the library yourself.

(4). POSIX thread library

Both libupnp and our WPS UPnP daemon need the POSIX thread library, following are recommended pthread library version.

For uCLibc, need the version >= 0.9.27

For GLIBC, need the version >= 2.3.2

If your pthread library is older than upper list, you may need to upgrade it.

Build and Run:

(1). Modify the “$(work_directory)/wsc_upnp/Makefile” and change the compile flags depends on your target platform.

Ex. For arm-linux target platform, you may need to set the following fags:

CROSS_COMPILE = arm-linux-

TARGET_HOST = arm-linux

WIRELESS_H_INCLUDE_PATH = kernel source include path

(2). Modify the “$(work_directory)/wsc_upnp/libupnp-1.3.1/Makefile.src” and change the configure parameters to make it suitable for your platform.

Ex. For big-endian system, you may need to add following CFAGS:

./configure --host=$(TARGET_HOST) --build=i386 CFLAGS="-mbig-endian"

(3). Compile it

Run “make” in “$(work_directory)/wsc_upnp”, after successful compilation, you will get an execution file named “wscd”.

(4). Install

Create a sub-directory named “xml” in the “/etc” of your target platform

Copy all files inside in “$(work_directory)/wsc_upnp/xml” to “/etc/xml”

Copy the “wscd” to the target platform.

(5). Run it

Before run it, be sure the target platform already has set the default route. Or the WPS daemon will failed when do initialization.

Now you can run it by following command:

/bin/wscd –m 1 –d 3

- Related Documents

(1). WPS Specification (Simple_Config_v1.0g.pdf)

(2). UPnP Device Architecture 1.0

(3). Windows Connect Now-NET Version 1.0

(3). WFAWLANConfig:1 Service Template Version 1.01

(4). WFA Device:1 Device Template Version 1.01