# 8.0  UART Module

CNIP provides a 16450 compatible UART, which contains 16 byte FIFOs. In addition, auto flow control is provided, in which, auto-CTS mode (CTS controls transmitter) and auto-RTS mode (Receiver FIFO contents and threshold control RTS) are both supported. The baud rate can be up to 1 Mbps and programmable baud rate generator allows division of any input reference clock by 1 to (2^16-1) and generates an internal 16x clock.  Berlin provides fully programmable serial interface, which can be configured to support 6,7,8 bit characters, even, odd, no parity generation and detection, and 1 or 2 stop bit generation. Last, fully prioritized interrupt control and loopback functionality for diagnostic capability are also provided.

## Note1.16C450 support 5,6,7,8 bit characters. And 1 , 3/2 , 2 stop bit generation.

## 8.1  Interface Pins

The UART interface pins are shown in Table 1.

Table 1:  The interface pins for UART0 control

| Signal Name | Type | Function | Note |
| --- | --- | --- | --- |
| TXD# | O | Transmit Data |  |
| RXD# | I | Receive Data |  |
| RTS# | O | Request To Send |  |
| CTS# | I | Clear To Send |  |

## UART Register Set ( Base Address: UART0=0xB800-2000, UART1=0xB800-2100

UART2=0xB800-2200, UART3=0xB800-2300)

Table 2: The Register Set of UART Control

(byte access only)

| Offset | Size (byte) | Name | Description | Access | Field |
| --- | --- | --- | --- | --- | --- |
| 000 | 1 | UART0_RBR | Receiver buffer register. (DLAB=0) | R | UART |
| 000 | 1 | UART0_THR | Transmitter holding register. (DLAB=0) | W | UART |
| 000 | 1 | UART0_DLL | Divisor latch LSB. (DLAB=1) | R/W | UART |
| 004 | 1 | UART0_DLM | Divisor latch MSB. (DLAB=1) | R/W | UART |
| 004 | 1 | UART0_IER | Interrupt enable register. (DLAB=0) | R/W | UART |
| 008 | 1 | UART0_IIR | Interrupt identification register. | R | UART |
| 008 | 1 | UART0_FCR | FIFO control register | W | UART |
| 00c | 1 | UART0_LCR | Line control register | R/W | UART |
| 010 | 1 | UART0_MCR | Modem control register | R/W | UART |
| 014 | 1 | UART0_LSR | Line status register | R | UART |
| 018 | 1 | UART0_MSR | Modem status register | R | UART |
| 01C | 1 | UART0_SCR | Scratch register | R/W | UART |

Table 3: The UART0 Receiver buffer register (DLAB=0)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 0:7 | RBR[0:7] | Receiver buffer data | R | 00H |

Table 4: The UART0 Transmitter holding register (DLAB=0)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 0:7 | THR[0:7] | Transmitter holding data | W | 00H |

Table 5: The UART0 Divisor latch LSB (DLAB=1)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 0:7 | DLL[0:7] | Divisor latch LSB | W/R | 00H |

Table 6: The UART0, Divisor latch MSB. (DLAB=1)

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 0:7 | DLM[0:7] | Divisor latch MSB | W/R | 00H |

Table 7: The UART0 Interrupt enable register

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 0 | ERBI | Enable received data available interrupt | R/W | 0 |
| 1 | ETBEI | Enable transmitter holding register empty interrupt | R/W | 0 |
| 2 | ELSI | Enable receiver line status interrupt | R/W | 0 |
| 3 | EDSSI | Enable modem status register interrupt | R/W | 0 |
| 4:7 |  | Reserved | R | 0000 |

Table 8: UART0 Interrupt identification register

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 0 | IPND | Interrupt pending 0 = interrupt pending | R | 0 |
| 3:1 | IID[2:0] | Interrupt ID. IID[1:0] indicates the interrupt priority. | R | 000 |
| 4:5 |  | Reserved | R | 00 |
| 6:7 | EFIFO EFIFO | FIFO enable : 00 = no FIFO 11 = 16-byte FIFO | R | 11 |

Table 9: UART0 FIFO control register

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 0 | EFIFO | Enable FIFO. When this bit is set, enable the transmitter and receiver FIFOs. Changing this bit clears the FIFOs. | W | 0 |
| 1 | RFRST | Receiver FIFO reset. Writes 1 to clear the receiver FIFO. | W | 0 |
| 2 | TFRST | Transmitter FIFO reset. Writes 1 to clear the transmitter FIFO. | W | 0 |
| 3 | DMODE | DMA mode. 0 = DMA mode 0 1 = DMA mode 1 | W | 0 |
| 7:6 | RECVTRG [1:0] | Receiver trigger level Trigger level: 16-byte 00 = 01,01 = 04, 10 = 08, 11 = 14 | W | 11 |

Note 2. 16C450 doesn’t support fifo control register, we can delete spec : changing efifo clears the fifos.

Table 10: UART0 Line control register

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 1:0 | WLS[1:0] | Word length select 00 = Reserved 01 = 6 bits 10 = 7 bits 11 = 8 bits | R/W | 1 |
| 1 |  |  | R | 1 |
| 2 | STB | Number of stop bits 0 = 1 bit 1 = 2 bits | R/W | 0 |
| 3 | PEN | Parity enable | R/W | 0 |
| 5:4 | {STKP,EPS } | Even parity select 00 = odd parity 01 = even parity 10 = mark parity 11 = space parity | R/W | 0 |
| 6 | BRCTL | Break control. Set this bit force TXD to the spacing (low) state.(break) Clear this bit to disable break condition. | R/W | 0 |
| 7 | DLAB | Divisor latch access bit | R/W | 0 |

Table 11: UART0 Modem control register

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 0 | DTR | Data terminal ready 0 = Set DTR# high 1 = Set DTR# low | R/W | 0 |
| 1 | RTS | Request to send 0 = Set RTS# high 1 = Set RTS# low | R/W | 0 |
| 2 | OUT1 | Out 1 | R/W | 0 |
| 3 | OUT2 | Out 2 | R/W | 0 |
| 4 | LOOP | Loopback | R/W | 0 |
| 5 | AFE | Auto flow control enable | R/W | 0 |
| 6 | LXCLK_SEL | Uart/timer clock select 0 = peripheral clk 1 = fixed 200MHz clk | W | 0 |

Table 12: UART0 Line status register

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 0 | DR | Data ready. Character mode: data ready in RBR FIFO mode: receiver FIFO is not empty. | R | 0 |
| 1 | OE | Overrun error. An overrun occurs when the receiver FIFO is full and the next character is completely received in the receiver shift register. An OE is indicated. The character in the shift register will be overwritten. | R | 0 |
| 2 | PE | Parity error | R | 0 |
| 3 | FE | Framing error | R | 0 |
| 4 | BI | Break interrupt indicator | R | 0 |
| 5 | TXEMT | Transmitter empty Character mode: both THR and TSR are empty. FIFO mode: both transmitter FIFO and TSR are empty | R | 0 |
| 6 | TXEMT |  | R | 0 |
| 7 | RFE | Errors in receiver FIFO. At least one parity, framing and break error in the FIFO. | R | 0 |

Table 13: UART0 Modem status register

| Bit | Bit Name | Description | R/W | InitVal |
| --- | --- | --- | --- | --- |
| 0 | R_DCD | Received DCD | R | 0 |
| 1 | R_RI | Received RI | R | 0 |
| 2 | R_DSR | Received DSR | R | 0 |
| 3 | R_CTS | Received CTS | R | 0 |
| 4 | D_DCD | Received DCD be triggered (rising or falling edge) | R | 0 |
| 5 | TERI | Received RI be triggered (falling edge) | R | 0 |
| 6 | D_DSR | Received DSR be triggered (rising or falling edge) | R | 0 |
| 7 | D_CTS | Received CTS be triggered (rising or falling edge) | R | 0 |

## 8.3  Baud Rate

(Value of divisor latch + 1) = Base clock/(16*bps)

## 8.4  Time-out interrupt

The time-out interrupt occurs when the following conditions exists at least one byte in the active descriptor buffer or receiver FIFO.

The most recent serial character was received more than four character times ago.

## 8.5  Auto-flow control

Auto-RTS: 	When the receiver FIFO reaches the trigger level, the RTS# is de-asserted. RTS# is automatically re-asserted once the FIFO is emptied. If RTS# is de-asserted, the UART must receive the incoming data until the FIFO is full.

Auto-CTS: 	The transmitter checks CTS# before sending the next data byte. When

CTS# is active, the transmitter sends the next data byte, otherwise it stop

the transmission.

## 8.6  Loopback diagnostic:

When LOOP bit is set, the following occurs.

TXD# assert high

RXD# disconnected

CTS#, DSR#, DCD# and RI# are disconnected

The output of the transmitter shift register is looped back into the input of the receiver shift register.

The MCR’s DTR, RTS, OUT1 and OUT2 bits are internally connected to CTS#, DSR#, DCD# and RI# respectively.

The DTR#, RTS#, OUT1# and OUT2# pins are forced high.

## 8.7  Interrupt priority

| Interrupt Identification Register |  |  |  | Priority level | Interrupt type | Interrupt source | Interrupt reset method |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit3 | Bit2 | Bit1 | Bit0 |  |  |  |  |
| 0 | 0 | 0 | 1 | None | None | None | None |
| 0 | 1 | 1 | 0 | 1 | Receiver line status | Overrun, parity, framing errors or break | Read LSR |
| 0 | 1 | 0 | 0 | 2 | Received data available | DR bit is set. | Read RBR. |
| 1 | 1 | 0 | 0 | 2 | Character time-out indication | No characters have been removed from or input to FIFO during the last character times and at 1 character in it. | Read RBR |
| 0 | 0 | 1 | 0 | 3 | Transmitter holding register empty | THRE bit set. | Reading IIR or write THR |
| 0 | 0 | 0 | 0 | 4 | Modem status | CTS#,DSR#,RI#,DCD# | Reading MSR |

Bit[7:6]={EFIFO,EFIFO}  Bit[5:4]={0,0}  Bit[0]={~uart_ip}

## 8.8  Break

A break characters is an all- zero character whose length is start bit (1) + word length (7 or 8) + parity bit (0 or 1) + stop bits (1 or 2). The break event occurs after the first break character is received.